Connor Mura

9/13/2026

CS 350

Milestone One Lab Questions

Lab Questions

- At what frequency can you see the LED start to blink?

The LED begins to noticeably blink around 20 Hz. Above 20 Hz, the blinking isn’t really noticeable and below 20 Hz the blinking becomes very distinct.

- At what duty cycle is the intensity of the LED perceptibly diminished from the initial 50% duty cycle?

The LED becomes noticeably dimmer when the duty cycle reaches about half, or roughly 25%. The diminishing brightness seems to be exponential, as it is initially imperceivable – but fades rapidly once the dimming becomes noticeable.

- When changing the duty cycle of the PWM, the loop used an increment of 5 every tenth of a second. Was this perceptibly smooth? If not, what could you change to improve the visual response? Why?

I would classify the change as smooth, yet not fluid. The transition isn’t choppy or jarring – but the exponential diminishing is noticeable. I would equate it more to the blinking one would find from anti-collision lights on tall towers than holiday decorations. If I wanted to capture that smoother, breathing-like blink I would change the step-frequency from 5 to 1 and the sleep time from 0.1 to 0.5. This would provide a fluid change at a slower pace.

- What function sets the PWM frequency for a GPIO line?

The function pwm18 = GPIO.PWM(18, 60) instantiates the PWM object, where the 18 represents the GPIO line and the 60 represents the frequency.

- What function sets the duty cycle for a GPIO line?

The method pwm18.start(0) initializes the duty cycle at 0% at the start of the PWM instance, and pwm18.ChangeDutyCycle(dc) dynamically updates the duty cycle within the loops.