Connor Mura

9/20/2026

CS-350

Milestone Two Lab Questions

1. Why do both the SerialTest-Write.py and SerialLightControl-Client.py scripts use the encode()

method of the string datatype when writing data to the serial port?

Serial communication interfaces transmit data as a stream of binary bytes, not as high level text. The encode method converts Python into byte format. This allows the data to be transmitted and prevents the serial write function from throwing a type error.

2. Why does the SerialTest-Read.py script use the decode() method of the string datatype when

reading the data from the serial port?

For the exact opposite reason talked about in the previous question. Data coming via serial interface is in binary byte format., in order for python to use said data it first needs to be converted back into string format.

3. What is the purpose of the try/except block in both the SerialLightControl-Client.py script and

the SerialLightControl-Server.py script?

Try/except blocks are a form of error handling. This block handles unexpected runtime interruptions – preventing system crashes and for embedded systems, ensuring the program runs cleanup before terminating.

4. Why is it necessary to make sure that the GPIO pins are always returned to their original state at the end of program run?

Returning GPIO pins to their original state, via cleanup, prevents electrical hazards and software bugs. Leaving pins in a high voltage state draws power which can overheat or short components. It also frees up hardware resources in the OS, preventing resource allocation lockouts.